Excellent, Mayuresh ✅ — these are classic Industrial IoT (IIoT) lab practicals combining IoT hardware, cloud communication, and data analytics concepts.

Below is a complete practical guide with:

- Explanation (Concept + Objective)

- Execution code (Arduino / Raspberry Pi / Python)

- Output / Expected Result

Let’s go through all 5 one by one 👇

⚙️ 1. Program for Sending Alert Messages to the User

Concept

In Industrial IoT, alert systems notify users about abnormal readings (e.g., high temperature, gas leaks, motion detection). Alerts can be sent via SMS, email, or push notifications.

Objective

To design a system that monitors an environmental variable and sends alerts to the user when a threshold is crossed.

Code (Python – Using MQTT + Email Alert)

import random

import smtplib

from email.mime.text import MIMEText

import time

THRESHOLD = 60  # Example temperature threshold

def send_alert(temp):

sender = "youremail@gmail.com"

receiver = "useremail@gmail.com"

msg = MIMEText(f"Alert! Temperature exceeded limit. Current temp: {temp}°C")

msg['Subject'] = "IIoT Alert Notification"

msg['From'] = sender

msg['To'] = receiver

try:

with smtplib.SMTP_SSL('smtp.gmail.com', 465) as server:

server.login("youremail@gmail.com", "your-app-password")

server.send_message(msg)

print("Alert sent successfully!")

except Exception as e:

print("Error sending alert:", e)

while True:

temp = random.randint(20, 80)  # Simulated temperature sensor data

print(f"Temperature: {temp}°C")

if temp > THRESHOLD:

send_alert(temp)

time.sleep(5)

Expected Output

- Displays temperature values on console.

- Sends an alert email when temperature exceeds 60°C.

⚙️ 2. Arduino / Raspberry Pi Program for Interfacing with PIR Sensor

Concept

A PIR (Passive Infrared) Sensor detects motion based on infrared radiation changes, useful in smart lighting and intrusion detection.

Objective

Interface a PIR sensor with Raspberry Pi or Arduino to detect human motion and trigger an LED or alert.

Arduino Code

int ledPin = 13;

int pirPin = 2;

int pirState = LOW;

int val = 0;

void setup() {

pinMode(ledPin, OUTPUT);

pinMode(pirPin, INPUT);

Serial.begin(9600);

}

void loop() {

val = digitalRead(pirPin);

if (val == HIGH) {

digitalWrite(ledPin, HIGH);

if (pirState == LOW) {

Serial.println("Motion detected!");

pirState = HIGH;

}

} else {

digitalWrite(ledPin, LOW);

if (pirState == HIGH) {

Serial.println("Motion ended!");

pirState = LOW;

}

}

}

Expected Output

- When motion is detected → LED turns ON and “Motion detected!” message prints.

- When no motion → LED turns OFF.

Raspberry Pi (Python) Code

import RPi.GPIO as GPIO

import time

PIR_PIN = 7

LED_PIN = 11

GPIO.setmode(GPIO.BOARD)

GPIO.setup(PIR_PIN, GPIO.IN)

GPIO.setup(LED_PIN, GPIO.OUT)

print("PIR Sensor Test (Ctrl+C to exit)")

time.sleep(2)

try:

while True:

if GPIO.input(PIR_PIN):

GPIO.output(LED_PIN, True)

print("Motion Detected!")

else:

GPIO.output(LED_PIN, False)

time.sleep(0.5)

except KeyboardInterrupt:

GPIO.cleanup()

⚙️ 3. Program to Design and Develop a User Interface for Monitoring and Controlling CPS System

Concept

Cyber-Physical Systems (CPS) integrate sensors, computation, and control. Here we design a web-based dashboard for monitoring and control.

Objective

Create a simple interface where users can view sensor data and control devices (like fans, lights).

Code (Python – Streamlit Dashboard)

import streamlit as st

import random

st.title("CPS Monitoring and Control Dashboard")

# Simulated sensor data

temperature = random.randint(20, 50)

humidity = random.randint(30, 90)

st.subheader("Sensor Data")

st.metric("Temperature (°C)", temperature)

st.metric("Humidity (%)", humidity)

st.subheader("Control Panel")

light = st.button("Toggle Light")

fan = st.button("Toggle Fan")

if light:

st.success("Light toggled successfully!")

if fan:

st.success("Fan toggled successfully!")

Expected Output

- Dashboard showing live temperature and humidity.

- Buttons to control devices (simulated toggling).

⚙️ 4. Program for Sending Sensor Data to Cloud and Displaying on Web Page

Concept

Sensors collect real-time data, which is uploaded to a cloud platform (ThingSpeak / Firebase / AWS IoT) for monitoring.

Objective

Send simulated or real sensor data to ThingSpeak cloud and visualize it via its web interface.

Code (Python – ThingSpeak Example)

import requests

import random

import time

WRITE_API_KEY = "YOUR_API_KEY"

THINGSPEAK_URL = "https://api.thingspeak.com/update"

while True:

temperature = random.uniform(20, 40)

humidity = random.uniform(40, 90)

data = {

'api_key': WRITE_API_KEY,

'field1': temperature,

'field2': humidity

}

response = requests.post(THINGSPEAK_URL, data=data)

print(f"Sent: Temp={temperature:.2f}°C, Humidity={humidity:.2f}% | Response: {response.status_code}")

time.sleep(15)

Expected Output

- Data continuously uploaded to ThingSpeak.

- Graphs visible on ThingSpeak dashboard in real-time.

⚙️ 5. Program for Performing Industrial Data Analysis

Concept

Industrial data analysis helps detect trends, predict maintenance needs, and optimize processes using analytics tools.

Objective

Analyze machine sensor data to find patterns and anomalies.

Code (Python – Pandas + Matplotlib + Scikit-learn)

import pandas as pd

import matplotlib.pyplot as plt

from sklearn.linear_model import LinearRegression

# Simulated industrial data

data = {

'Temperature': [50, 55, 60, 65, 70, 75],

'Vibration': [3.2, 3.4, 3.6, 3.9, 4.2, 4.5],

'MotorHealth': [95, 90, 85, 75, 60, 45]

}

df = pd.DataFrame(data)

print(df)

# Visualization

plt.plot(df['Temperature'], df['MotorHealth'], marker='o')

plt.title("Motor Health vs Temperature")

plt.xlabel("Temperature (°C)")

plt.ylabel("Motor Health (%)")

plt.show()

# Linear regression to predict motor health

model = LinearRegression()

X = df[['Temperature']]

y = df['MotorHealth']

model.fit(X, y)

temp = 80

predicted_health = model.predict([[temp]])

print(f"Predicted motor health at {temp}°C = {predicted_health[0]:.2f}%")

Expected Output

- Displays table of simulated data.

- Graph shows health degradation with temperature.

- Model predicts motor health at any given temperature.

✅ Summary Table

| Practical | Focus | Technology | Output |
| --- | --- | --- | --- |
| 1 | Alert Messaging | Python + SMTP | Email alerts |
| 2 | PIR Sensor | Arduino / RPi | Motion detection |
| 3 | CPS Dashboard | Streamlit UI | Monitoring & control |
| 4 | Cloud Integration | Python + ThingSpeak | Real-time web visualization |
| 5 | Industrial Data Analysis | Pandas + ML | Predictive insights |

Would you like me to now compile these 5 practicals into a formatted PDF lab record (with “Aim, Apparatus, Procedure, Code, Output, Result” sections for submission)?